STYRENE PARK OFFSITE Inspection and Test Plan (ITP) for Ru0001A / B-D-02

| R2 | 15-Apr.-2025 | IFA | F.Malekifar | M.Yasini | GH.Azizi |
| --- | --- | --- | --- | --- | --- |
| R1 | 24-Nov.-2024 | IFA | F.Malekifar | M.Yasini | GH.Azizi |
| R0 | 05-Nov.-2024 | IFA | F.Malekifar | M.Yasini | GH.Azizi |
| Rev. | Issued Date | DESCRIPTION | PREPARED | CHECKED | APPROVED |

REVISION RECORD SHEET

| Page Page | Revisions |  |  |  |  |  |  |  | Page | Revisions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | R0 | R1 | R2 | R3 | R4 | R5 | R6 |  |  | R0 | R1 | R2 | R3 | R4 | R5 | R6 |
| 1 | X | X | X |  |  |  |  |  | 41 |  |  |  |  |  |  |  |
| 2 | X | X | X |  |  |  |  |  | 42 |  |  |  |  |  |  |  |
| 3 | X | X |  |  |  |  |  |  | 43 |  |  |  |  |  |  |  |
| 4 | X | X |  |  |  |  |  |  | 44 |  |  |  |  |  |  |  |
| 5 | X | X |  |  |  |  |  |  | 45 |  |  |  |  |  |  |  |
| 6 | X | X | X |  |  |  |  |  | 46 |  |  |  |  |  |  |  |
| 7 | X | X |  |  |  |  |  |  | 47 |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  | 48 |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  | 49 |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |  | 51 |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |  |  | 52 |  |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |  |  | 53 |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |  |  | 54 |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |  | 55 |  |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |  |  | 56 |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |  |  | 57 |  |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |  |  | 58 |  |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |  |  | 59 |  |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |
| 21 |  |  |  |  |  |  |  |  | 61 |  |  |  |  |  |  |  |
| 22 |  |  |  |  |  |  |  |  | 62 |  |  |  |  |  |  |  |
| 23 |  |  |  |  |  |  |  |  | 63 |  |  |  |  |  |  |  |
| 24 |  |  |  |  |  |  |  |  | 64 |  |  |  |  |  |  |  |
| 25 |  |  |  |  |  |  |  |  | 65 |  |  |  |  |  |  |  |
| 26 |  |  |  |  |  |  |  |  | 66 |  |  |  |  |  |  |  |
| 27 |  |  |  |  |  |  |  |  | 67 |  |  |  |  |  |  |  |
| 28 |  |  |  |  |  |  |  |  | 68 |  |  |  |  |  |  |  |
| 29 |  |  |  |  |  |  |  |  | 69 |  |  |  |  |  |  |  |
| 30 |  |  |  |  |  |  |  |  | 70 |  |  |  |  |  |  |  |
| 31 |  |  |  |  |  |  |  |  | 71 |  |  |  |  |  |  |  |
| 32 |  |  |  |  |  |  |  |  | 72 |  |  |  |  |  |  |  |
| 33 |  |  |  |  |  |  |  |  | 73 |  |  |  |  |  |  |  |
| 34 |  |  |  |  |  |  |  |  | 74 |  |  |  |  |  |  |  |
| 35 |  |  |  |  |  |  |  |  | 75 |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |  | 76 |  |  |  |  |  |  |  |
| 37 |  |  |  |  |  |  |  |  | 77 |  |  |  |  |  |  |  |
| 38 |  |  |  |  |  |  |  |  | 78 |  |  |  |  |  |  |  |
| 39 |  |  |  |  |  |  |  |  | 79 |  |  |  |  |  |  |  |
| 40 |  |  |  |  |  |  |  |  | 80 |  |  |  |  |  |  |  |

- Purpose

The  inspection  and  test  plan  is  an  engineering  document,  which  defines  the  type  and extent of CONTRACTOR and CLIENT involvement in each phase of fabrication, control and testing requirements and inspection.

- Applicable Documents

| ASME Sec. II |  | Material Rules for Construction of Pressure Vessels Rules for Construction of Pressure Vessels, Alternative Rules Non-Destructive Examination Welding and Brazing Qualifications Material Certificates |
| --- | --- | --- |
| ASME Sec. VIII Div. 1 |  |  |
| ASME Sec. VIII Div. 2 |  |  |
| ASME Sec .V ASME Sec. IX |  |  |
| BS EN 10204 (3.1) |  |  |
| ASME Sec .V ASME Sec. IX |  |  |
| All Standards and Specifications listed in the Requisition |  |  |

- Defenitions

| Abbr. | Description |
| --- | --- |
| PROJECT | Receiver Header/ PR200 |
| Client / TPI | HSE |
| (C) CONTRACTOR | Toase-ehe Park Sanati Gohar Ofogh |
| (V) Vendor | Arkan Sanat Paydar Company(ASP) |

- Definition of CONTRACTOR and CLIENT / TPI Involvement

The  nature  of  CONTRACTOR  and  CLIENT involvement  is  indicated  against  each activity  of  fabrication  and  testing  by  means  of  the  letters  H,  W,  SW,  R  and  A  with  the following definitions:

H: (Hold) Point

The VENDOR cannot carry out the specified controls and tests without CONTRACTOR and  CLIENT  attendance  and  witnessing.  The VENDOR  must  notify  CONTRACTOR  by fax  of  the  dedicated  inspection  activity  at least  twenty  (20)  Calendar  days  in  advance and consequently CLIENT will be informed by CONTARACTOR. The VENDOR cannot deviate from this rule unless written approval has been given by CONTRACTOR.

W:(Witness)

The  VENDOR  must  notify  CONTRACTOR  of  the  dedicated  inspection  activity  at  least twenty  (20)  Calendar  days  in  advance  and consequently  CLIENT  will  be  informed  by CONTARACTOR. Although CONTRACTOR and CLIENT witnessing is not mandatory.

If CONTRACTOR does not elect to be present, the VENDOR may proceed with his own inspection, provided controls and test records are made available to CONTRACTOR for review.

SW: (Spot Witness)

Spot Witness denotes to random attendance of inspector without any invitation from the VENDOR and without advance notification by the CONTRACTOR to the VENDOR.

R: (Review) – Review of Documents

The  VENDOR  has  either  to  submit  to  CONTRACTOR  for  review  and  comments  the documents  required  prior  to  the  performance  of the  dedicated  activity  or  to  transmit  or make  available  for  the  review  of  CONTRACTOR  the  result  of  the  controls  and  tests conducted,  as  the  case  may  be.  Consequently  CLIENT  will  be  informed  by CONTARACTOR.

A: (Approval) – Approval of Documents

The VENDOR has either to submit to CONTRACTOR for comments and approvals the documents  required  prior  to  the  performance  of the  dedicated  activity  or  to  transmit  or make  available  for  the  approval  of  CONTRACTOR  the  result  of  the  controls  and  tests conducted,  as  the  case  may  be.  VENDOR  is  not  allowed  to  proceed  the  work  before receipt of CONTRACTOR approval on the required documents.

Table: Inspection Activities

| ROWS | INSPECTION ACTIVITIES | APPLICABLE DOCUMENTS | VENDOR | CONTRACTOR | CLIENT / TPI |
| --- | --- | --- | --- | --- | --- |
| 1 | VENDOR’s Quality Control Plan | VENDOR’s Document | H | R | R |
| 2 | Pre-Inspection Meeting | Agenda | H | H | H |
| 3 | Material Test Certificate | BS EN 10204 (3.1) | H | R | R |
| 4 | Checking The Material With Requirements of Requisition | Requisition | H | W | W |
| 5 | Inspection of Sub Ordered Components | Requisition | H | W | R |
| 6 | Material identifications and markings | - | H | W | W |
| 7 | Storage of materials and welding consumables | - | H | W | R |
| 8 | WPS/PQR | Design Code,ASME Sec IX | H | R/A | R |
| 9 | Production Test Coupons (if required) | Design Code,ASME Sec IX | H | R/A | R |
| 10 | NDT Personnel’s Qualification | Design Code,ASME Sec V,ASNT Lev.II or eqv. | H | R/A | R |
| 11 | Material Traceability | - | H | R | R |
| 12 | Dimensional check (before forming &rolling) | Approved Drawings | H | W | R |
| 13 | Cutting, Forming, Rolling, Punching,Drilling, Tacking and Fit up | Approved Drawings | H | W | R |
| 14 | Inspection of Back Gouged portions of edges prepared for welding by MT or PT | Design Code, ASME Sec V | H | W | W |
| 15 | Welding | Design Code,ASME Sec IX | H | W | W |
| ROWS | INSPECTION ACTIVITIES | APPLICABLE DOCUMENTS | VENDOR | CONTRACTOR | CLIENT / TPI |
| 16 | Confirmation of NDT Results RT, UT,MT, PT (Review of all Radiographs) | Design Code, ASME Sec V | H | W/A | W |
| 17 | Stress Relieving on dished head | ASME Sec VIII Div 1 , Approved Drawings, Approved PWHT Procedure | H | R | R |
| 18 | MT on Bevel Cut Edge | ASME Sec VIII Div 1, Approved MT Procedure | H | R | R |
| 19 | Mt/100 on Knuckel & Crown Areas | ASME Sec VIII Div 1, Approved MT Procedure | H | R | R |
| 20 | Inspection After dish forming- UT thickness verification , Dimensional & Visual Check | Approved Drawings | H | R | R |
| 21 | Stress Relieving on dished head | ASME Sec VIII Div 1 – USC-56, Approved PWHT procedure | H | R | R |
| 22 | Execution of Major Repair | Requisition | H | W | W |
| 23 | Completeness check | Approved Drawings | H | H | H |
| 24 | Roughness checking (Flange Facing,Internal Surface, etc.) | Approved Drawings | H | W | R |
| 25 | Confirmation of Hardness Measurement in Final Condition | Design Code, ASME Sec V | H | W | R |
| 26 | Final Visual and Dimensional Check | Approved Drawings | H | H | H |
| 27 | Reinforcing Pad air-soap test | Requisition | H | W | W |
| 28 | Leakage Test/Hydrotest | Requisition | H | H | H |
| 29 | Complete Draining and drying,N2/VPT/desiccant filling, (which applicable) | Requisition | H | W | W |
| 30 | Surface Preparation, Anti-Corrosion /Painting | Requisition | H | W | W |
| 31 | Name plate, Cleanliness, Tagging and Marking | Requisition | H | W | W |
| ROWS | INSPECTION ACTIVITIES | APPLICABLE DOCUMENTS | VENDOR | CONTRACTOR | CLIENT / TPI |
| 32 | Packing / Preparation for Shipment | Requisition | H | W | W |
| 33 | Check Identification and Destination Marking | Requisition | H | W | - |
| 34 | Final Inspection Prior to Shipment | Requisition | H | H | W |
| 35 | Non Conformities Treatment | Requisition | H | H | W |
| 36 | NDT Documents | Design Code, ASME Sec V | H | R/A | R |
| 37 | Documentations | Requisition | H | A | W/R |
| 38 | Control of Packing List | - | H | H | - |
| 39 | Deviations | - | H | H | W/R |
| 40 | Final Inspection / Data Book | Requisition | H | H/A | W/R |
| 41 | Release Note | - | H | H | H |